SANFORD SEWERAGE DISTRICT 
PRETREATMENT RULES AND REGULATIONS

Approved on April 13,2015

Proposed local limits relaxation by the September 14, 2015 following a DEP comment period of 30 days- Oct. 26, 2015

Approved on January 9, 2017

Approved on January 8, 2018

Approved on August 19, 2019

TABLE OF CONTENTS

Page

Article I. Abbreviations and Definitions	1	Section 1.  Abbreviations	1		Section 2.  Definitions	1

Article II. General Sewer Use Requirements	10		Section 1. Prohibited Discharge Standards	10

Section 2. Bypass	13

Article III. Rates, Service Charges	14

Article IV. Commercial and Industrial Discharge to Public Sewers Program	14

Section 1. Pretreatment Purpose and Policy	14		Section 2. Administration	14

Section 3. District Review of Pretreatment Facility Plans	15

Section 4. Prohibited Wastes and Wastewaters	15

Section 5. Fats, Oils, and Grease (FOG)	15		Section 6. National Categorical Pretreatment Standards	18

Section 7. Pretreatment Facilities	19		Section 8. State Pretreatment Standards	19	Section 9. Records and Public Notification Procedures	19	Section 10. District’s Right of Revision	20	Section 11. Excessive Discharge	20	Section 12. Discharges	21	Section 13. Fees	21		Section 14. Pretreatment Procedures (Permitting, Compliance Tracking and	22

Enforcement)

Section 15. Accidental Discharge/Slug Discharge Control Plans	25

Section 16. Hauled Wastewater	25

Section 17. Regulation of Waste Received from Other Jurisdictions	26

Section 18. Permits	26

Section 19. Reports of Changed Conditions	34

Section 20. Special Agreements	34

Section 22. Indemnification	34

Article V.  Enforcement and Appeals	35

Article VI. Protection from Damage	38

Article VII. Powers and Authority of Inspectors	38

Article VIII. Severability	39

Article IX.  Validity	39

Article X. Effective Date	39

SANFORD SEWERAGE DISTRICT’S

PRETREATMENT RULES AND REGULATIONS

Be it ordained and adopted on the date last stated by the Sewer District of the City of Sanford, County of York, State of Maine, these  “Rules and Regulations Governing the Use of the Public and Private Sewers” (hereinafter the “Rules and Regulations), governing the use of public and private sewers and drains, the installation and connection of building sewers, the discharge of wastes and waters into the public sewer system(s), and providing penalties for violations thereof in the City of Sanford, County of York, and State of Maine, as follows:

Article I – Abbreviations and Definitions

Section 1 – Abbreviations

The following abbreviations, when used in these Rules and Regulations, shall have the designated meanings:

BOD – Biochemical Oxygen Demand

BMP – Best Management Practice

BMR – Baseline Monitoring Report

CFR – Code of Federal Regulations

CIU – Categorical Industrial User

COD – Chemical Oxygen Demand

DISTRICT OR SSD – Sanford Sewer District

EPA – U.S. Environmental Protection Agency

FOG – Fats, Oil and Grease

gpd – gallons per day

mg/l – milligrams per liter

MEDEP – Maine Department of Environmental Protection

MEPDES – Maine Pollutant Discharge Elimination System

NSCIU – Non-Significant Categorical Industrial User

POTW – Publicly Owned Treatment Works

RCRA – Resource Conservation and Recovery Act

SIU – Significant Industrial User

SNC – Significant Noncompliance

TSS – Total Suspended Solids

U.S.C. – United States Code

User – Industrial User

Section 2 - Definitions

As used in these Pretreatment Rules and Regulations, defined terms are capitalized, and other terms shall have their commonly understood meanings. Unless the context specifically indicates otherwise, the meaning of terms used in these Pretreatment Rules and Regulations shall be as follows:

Act or “the Act.”  Shall mean the Federal Water Pollution Control Act, also known as the Clean Water Act, as amended, 33 U.S.C. section 1251 et seq.

Approval Authority. Shall mean the Maine Department of Environmental Protection (MDEP).

Authorized or Duly Authorized Representative of the User.

(1)		If the User is a corporation:

(a)	The president, secretary, treasurer, or a vice-president of the corporation in charge of a principal business function, or any other person who performs similar policy or decision-making functions for the corporation; or

(b) 	The manager of one or more manufacturing, production, or operating facilities, provided the manager is authorized to make management decisions that govern the operation of the regulated facility including having the explicit or implicit duty of making major capital investment recommendations, and initiate and direct other comprehensive measures to assure long-term environmental compliance with environmental laws and regulations; can ensure that the necessary systems are established or actions taken to gather complete and accurate information for individual wastewater discharge permit requirements; and where authority to sign documents has been assigned or delegated to the manager in accordance with corporate procedures.

(2)		If the User is a partnership or sole proprietorship: a general partner or proprietor, respectively.

(3) 	If the User is a Federal, State, or local governmental facility:  a director or highest official appointed or designated to oversee the operation and performance of the activities of the government facility, or their designee.

(4) 		The individuals described in paragraphs 1 through 3, above, may designate a Duly Authorized Representative if the authorization is in writing, the authorization specifies the individual or position responsible for the overall operation of the facility from which the discharge originates or having overall responsibility for environmental matters for the company, and the written authorization is submitted to the Sanford Sewerage District.

Biochemical Oxygen Demand or BOD.  Shall mean the quantity of oxygen utilized in the biochemical oxidation of organic matter under standard laboratory procedures for five (5) days at 20 degrees centigrade, usually expressed as a concentration (e.g., mg/l).

Best Management Practices or BMPs means schedules of activities, prohibitions of practices, maintenance procedures, and other management practices to implement the prohibitions [40 CFR 403.5(a)(1) and (b)]. BMPs include treatment requirements, operating procedures, and practices to control plant site runoff, spillage or leaks, sludge or waste disposal, or drainage from raw materials storage.  BMPs also include alternative means of complying with, or in place of certain established categorical Pretreatment Standards and effluent limitations.

Categorical Pretreatment Standard or Categorical Standard.  Shall mean any regulation containing pollutant discharge limitation promulgated by EPA in accordance with section 307(b) and (c) of the Act (33 U.S.C. §1317) that apply to a specific category of Users and that appear in 40 CFR Chapter I, Subchapter N, Parts 405-471.

Categorical Industrial User.  Shall mean an Industrial User subject to a categorical pretreatment standard or categorical standard.

Chemical Oxygen Demand (COD).  Shall mean a quantity of dissolved oxygen required for the chemical oxidation of decomposable matter under aerobic conditions.

Composite Sample.  Shall be time-composite sample collected using automatic sampling equipment or a minimum of eight (8) equal volume grab samples collected over equal time intervals for the total period of discharge not to exceed 24 hours.

Control Authority.  Shall be the Sanford Sewerage District

Daily Maximum.  Shall be the arithmetic average of all effluent samples for a pollutant collected during a calendar day.

Daily Maximum Limit.  Shall be the maximum allowable discharge limit of a pollutant during a calendar day.  Where Daily Maximum Limits are expressed in units of mass, the daily discharge is the total mass discharged over the course of the day.  Where Daily Maximum Limits are expressed in terms of a concentration, the daily discharge is the arithmetic average measurement of the pollutant concentration derived from all measurements taken that day.

Direct Discharge. Shall mean the discharge of treated or untreated wastewater directly to the waters of the State of Maine.

Discharger. Shall mean any Person who contributes, causes or permits the contribution of Wastewater into the District facilities.

District. Shall mean the Sanford Sewerage District acting through its Board of Trustees, Superintendent or other duly authorized agent.

District Facilities. Shall include structures, conduits, pump stations, wastewater collection, treatment and disposal facilities, and other appurtenances for the purpose of collecting, treating and disposal of Domestic and/or Industrial Wastewater owned by the District.

Easement. Shall mean a perpetual right for the specific use of land where the fee interest is held by others.

Effluent. Shall mean the discharge of flow from a reservoir, basin, treatment process or treatment facility.

Environmental Protection Agency (or EPA).  The U.S. Environmental Protection Agency or, where appropriate, the term may also be used as designation for the Administrator or other duly authorized official of said agency.

Excessive. Shall mean amounts or concentrations of a constituent of a Sanitary or Industrial Wastewater which in the judgment of the District: (a) will cause damage to any facility, (b) will be harmful to a wastewater treatment process, (c) cannot be removed in the District’s Facilities to the degree required in the District’s Maine Pollutant Discharge Elimination System (MEPDES) permit, (d) can otherwise endanger life or property, or € can constitute a nuisance.

Existing Source.  Shall mean any source of discharge that is not a “New Source.”

Floatable Oil. Shall mean oil, fat, or grease in a physical state such that it will separate by gravity from wastewater by treatment in an approved pretreatment facility. Wastewater shall be considered free of floatable oil if it is properly pretreated and the wastewater does not interfere with the collection system.

Food Service Establishment (FSE).  Shall mean any place that prepares, services or handles food for consumption by the public, its members or employees and which has any process or device that uses or produces fats, oil and grease; or grease vapors, steam, fumes, smoke or odors that are required to be removed by a hood as defined in the Universal Plumbing Code.

Garbage.  Shall mean solid wastes from domestic and commercial preparation, cooking and dispensing of food and from the handling of storage and sale of produce.

Grab Sample.  Shall mean a sample that is taken from a waste stream without regard to the flow in the waste stream and over a period of time not to exceed fifteen (15) minutes.

Grease. Shall mean the material removed from a grease interceptor (or trap) serving a restaurant or other facility requiring such grease interceptors. Also means volatile and non-volatile residual fats, fatty acids, soaps, waxes and other similar materials.

Grease Interceptor/Grease Trap. Shall mean a plumbing appurtenance or appliance that is installed in a sanitary or wastewater discharge system to intercept nonpetroleum fats, oils and grease (FOG) from a waste water discharge.

Holding Tank. Shall mean wastewater holding tanks that are accessory to or part of vessels, chemical toilets, campers, trailers, recreational vehicles, septic tanks, dwelling units, and vacuum-pump tank trucks.

Incompatible Pollutant. Shall mean any Pollutant other than biochemical oxygen demand, suspended solids, pH, coliform bacteria, or additional pollutants identified in the permit, which the POTW was not designed to treat and do not remove to a substantial degree.

Indirect Discharge. The discharge or the introduction of Non-domestic Pollutants from any source regulated under Section 307(b) or (c) of the Act, (33 U.S.C. 1317), into the District Facilities (including holding tank waste discharged into the District Facilities)

Instantaneous Limit.  The maximum concentration of a pollutant allowed to be discharged at any time, determined from the analysis of any discrete or composited sample collected, independent of the industrial flow rate and the duration of the sampling event.

Interference.  A discharge that, alone or in conjunction with a discharge or discharges from other sources, inhibits or disrupts the POTW, its treatment processes or operations or its sludge processes, use or disposal; and therefore, is a cause of a violation of the District’s  MEPDES permit or of the prevention of sewage sludge use or disposal in compliance with any of the following statutory/regulatory provisions or permits issued thereunder, or any more stringent State or local regulations:  section 405 of the Act; the Solid Waste Disposal Act, including Title II commonly referred to as the Resource Conservation and Recovery Act (RCRA); any State regulations contained in any State sludge management plan prepared pursuant to Subtitle D of the Solid Waste Disposal Act; the Clean Air Act; the Toxic Substances Control Act; and the Marine Protection, Research, and Sanctuaries Act.

Local Limits.  Specific discharge limits developed and enforced by the Sanford Sewerage District upon industrial or commercial facilities to implement the general and specific discharge prohibitions listed in 40 CFR 403.5(a) (1) and (b).

Maine Pollution Discharge Elimination System Permit or MEPDES. Shall mean a permit issued pursuant to the Federal Water Pollution Control Act, Title 33 USC, Section 1251, et seq. and Conditions of Licenses, 38 M.R.S.A, Section 414-A et seq., and applicable regulations.

May is permissive (see “Shall”).

Medical Waste.  Isolation wastes, infectious agents, human blood and blood products, pathological wastes, sharps, body parts, contaminated bedding, surgical wastes, potentially contaminated laboratory wastes, and dialysis wastes.

Monthly Average.  The sum of all “daily discharges” measured during a calendar month divided by the number of “daily discharges” measured during that month.

Monthly Average Limit. The highest allowable average of “daily discharges” over a calendar month, calculated as the sum of all “daily discharges” measured during a calendar month divided by the number of “daily discharges” measured during that month.

Natural Outlet.  Shall mean any outlet into a watercourse, pond, ditch, lake or other body of surface or groundwater.

New Source.  Shall have the meaning set forth in 40 CFR § 403.3 (k) as amended, which states that it means:

(1)	Any new Building Sewer;

(2) 	Any building, structure, facility or installation from which there is or may be a Discharge of Pollutants, the construction of which commenced after the publication of proposed Pretreatment Standards under Section 307(c) of the Act which will be applicable to such source if such standards are thereafter promulgated in accordance with that section, provided that:

a)	The building structure, facility or installation is constructed at a site at which no other source is located; or

b)	The building, structure, facility or installation totally replaces the process or production equipment that causes the Discharge of Pollutants at an Existing Source; or

c)	The production or Wastewater generating processes of the building, structure, facility or installation are substantially independent of an Existing Source at the same site. In determining whether these are substantially independent, factors such as the extent to which the new facility is engaged in the same general type of activity as the Existing Source shall be considered.

(3)	Construction on a site at which an Existing Source is located results in a modification rather than a New Source if the construction does not create a new building, structure, facility or installation meeting the criteria of Section 2 (b) or (c) above but otherwise alters, replaces or adds to existing process or production equipment.

(4)	Construction of a New Source as defined under this Section has commenced if the Owner or operator has:

a)	Begun or caused to begin, as part of a continuous on-site construction program:

i.	Any placement, assembly or installation of facilities or equipment; or

ii.	Significant site preparation work including clearing, excavation or removal of existing buildings, structures or facilities which is necessary for the placement, assembly or installation of new source facilities or equipment; or

b)	Entered into a binding contractual obligation for the purchase of facilities or equipment which are intended to be used in its operation within a reasonable time. Options to purchase or contracts which can be terminated or modified without substantial loss, and contracts for feasibility, engineering and design studies do not constitute a contractual obligation under this paragraph.

Non-domestic Pollutant. Shall mean any substance other than human excrement or household gray water.

Noncontact Cooling Water.  Water used for cooling that does not come into direct contact with any raw material, intermediate product, waste product, or finished product.

Pass Through.  Shall have the meaning set  forth in 40 CFR § 403.3 as amended, which states that it means a discharge which exits the POTW into waters of the United States in quantities or concentrations which, alone or in conjunction with a discharge or discharges from other sources, is a cause of a violation of any requirement of the POTW’s MEPDES Permit (including an increase in the magnitude or duration of the violation).

Person.  Shall mean any individual, partnership, co-partnership, firm, company, corporation, association, joint stock company, trust, estate, governmental entity, or any other legal entity; or their legal representatives, agents, or assigns.  This definition includes all Federal, State, and local governmental entities.

pH.  Shall mean the logarithm of the reciprocal of weight of hydrogen ions in grams per liter of solution. It is a method of expressing the acidic or basic strength of a solution and the tendency or ability of that solution to react with other acidic or basic solutions.

Pollutant or Pollutants. Shall have the meaning set forth in 40 CFR § 401.11 as amended, which states in part that it means dredged spoil, solid waste, incinerator residue, sewage, garbage, sewage sludge, munitions, chemical wastes, biological materials, radioactive materials, heat, wrecked or discarded equipment, rock, sand, cellar dirt, and industrial, municipal and agricultural waste discharged into water.

Pretreatment.  Shall mean the reduction of the amount of Pollutants, the elimination of Pollutants, or the alteration of the nature of Pollutant properties in Wastewater prior to, or in lieu of, introducing such pollutants into the POTW. This reduction or alteration can be obtained by physical, chemical, or biological processes; by process changes; or by other means, except by diluting the concentration of the pollutants unless allowed by an applicable Pretreatment Standard.

Pretreatment Requirements.  Shall mean any substantive or procedural requirement related to Pretreatment imposed on a User, other than a Pretreatment Standard.

Pretreatment Standards.  Shall mean prohibited discharge standards, Categorical Pretreatment Standards, and Local Limits established by order of the Board of Trustees.

Private Sewage Disposal System.  Shall mean a subsurface waste water disposal system or septic system.

Private Sewer System or Private Sewer. Shall mean any sewer, not owned by the District, that collects wastewater from two or more building sewers and discharges to a public sanitary sewer. Private sewer systems are not permitted except by specific agreement with the District.

Prohibited Discharge Standards or Prohibited Discharges.  Shall mean absolute prohibitions against the discharge of certain substances as identified in Section 1 of these Rules and Regulations.

Public Sewer. Shall mean a sewer that is controlled by the Sanford Sewer District.

Publicly Owned Treatment Works or POTW.  Shall mean the Sanford Sewerage District’s devices or systems used in the collection, storage, treatment, recycling and reclamation of sewage or industrial wastes of a liquid nature. It also includes sewer, pipes, and other conveyances only I these structures convey wastewater to the POTW wastewater treatment facility.

Sanitary Sewer. Shall mean a Sewer which carries domestic, Sanitary and/or Industrial Wastewater sewage.

Sanitary Wastewater or Sewage. Shall mean the liquid waste discharge from a building’s or structure’s sanitary conveniences, such as toilets, washrooms, urinals, sinks, showers, and small laundries free of Industrial Wastes or toxic materials. Sanitary Wastewater may or may not be discharged separately from Industrial Wastewater. For a combined discharge the District shall determine if a wastewater discharge meets the definition “sanitary wastewater”.

Septage. Shall mean all sludge, scum, liquid, or any other material removed from a septic tank or disposal field.

Sewer. Shall mean a pipe or conduit for carrying sewage.

Sewer Extension. Shall mean any newly constructed sewer that is to be adopted and maintained by the District or privately maintained but connected to the District Facilities.

Significant Industrial User (SIU) is:

(1) 	An Industrial User subject to categorical Pretreatment Standards; or

(2) 	An Industrial User that:

a)	 	Discharges an average of twenty-five thousand (25,000) gpd or more of process wastewater to the POTW (excluding sanitary, noncontact cooling and boiler blowdown wastewater);

b) 	Contributes a process waste stream which makes up five (5) percent or more of the average dry weather hydraulic or organic capacity of the POTW treatment plant; or

c) 	Is designated as such by the District on the basis that it has a reasonable potential for adversely affecting the POTW’s operation or for violating any Pretreatment Standard or Requirement, the District may at any time, on its own initiative or in response to a petition received from an Industrial User, and in accordance with procedures in 40 CFR 403.8(f)(6), determine that such User should not be considered a Significant Industrial User.

Shall is mandatory.

Slug Load or Slug Discharge.  Shall be any discharge at a flow rate or concentration, which could cause a violation of the prohibited discharge standards in Section 1 of these Pretreatment Rules and Regulations.  A Slug Discharge is any Discharge of a non-routine, episodic nature, including but not limited to an accidental spill or a non-customary batch Discharge, which has a reasonable potential to cause Interference or Pass Through, or in any other way violate the POTW’s regulations, Local Limits or Permit conditions.

Standard Methods. Shall mean Standard Methods for the Examination of Water and Wastewater, latest edition, published by the American Public Health Association.

Storm Water.  Any flow occurring during or following any form of natural precipitation, and resulting from such precipitation, including snowmelt.

Superintendent. Shall mean the Superintendent of the Sanford Sewerage District or his authorized deputy, agent or representative.

Suspended Solids.  Shall mean solids that either float on the surface or are in suspension in water, sewage or other liquids and which are removable by laboratory filtering.

Toxic Pollutant. Any pollutant or combination of pollutants listed as toxic in regulations promulgated by the Administrator of the Environmental Protection Agency under the provision of CWA 307(a) or other acts.

Trustees or Board of Trustees. Shall mean the Trustees of the Sanford Sewerage District or their authorized agent or representative.

User or Industrial User.  A source of indirect discharge.

Wastewater, Wastewater Discharge, or Wastes.  Shall mean the used water and water-carried solids from the community that flow into the District Facilities.

Article II—General Sewer Use Requirements

Section 1 –  Prohibited Discharge Standards

General Prohibitions.  No Person/User shall introduce or cause to be introduced into the POTW any pollutant or wastewater which causes Pass Through or Interference.  These general prohibitions apply to all Users of the POTW.

Specific Prohibitions.  No Person/User shall introduce or cause to be introduced into the POTW the following pollutants, substances, or wastewater if it appears likely in the opinion of the District that such Wastewater can harm either the sewers, sewage treatment process or equipment, have an adverse effect on the receiving stream, or can otherwise endanger life, limb, public property or constitute a nuisance. In forming an opinion as to the acceptability of these Wastewaters, the District will give consideration to such factors as the quantities of subject Wastewaters in relation to flows and velocities in the sewers, materials of construction of the sewers, nature of the sewage treatment process, capacity of the POTW, degree of treatability of wastes in the sewage treatment and other pertinent factors, the substances prohibited are:

1)	Any liquids, solids, or gases which by reason of their nature or quantity are, or may be, sufficient either alone or by interaction with other substances to cause fire or explosion or be injurious in any other way to the District Facilities or to the operation of the District’s POTW. At not time shall two successive readings on an explosion hazard meter at the point of discharge into the District Facilities (or at any point in the District Facilities) be more than five percent (5%) nor any single reading over ten percent (10%) of the lower explosive limit (LEL) of the meter. Prohibited materials include, but are not limited to: gasoline, kerosene, naphtha, benzene, toluene, xylene, ethers, alcohols, ketones, aldehydes, peroxides, chlorates, perchlorates, bromates, carbides, hydrides, and sulfides and any other substances which the District, the State or EPA has notified the User is afire hazard or a hazard to the District Facilities.

2)  	Any Wastes or Wastewaters containing toxic or poisonous solids, liquids or gases in sufficient quantity, either singly or by interaction with other wastes, to injure or interfere with any POTW sewage treatment process, constitute a hazard to humans or animals, create a public nuisance, create any hazard in the receiving waters of the POTW, or to exceed the limitation set forth in a Categorical Pretreatment Standard. A toxic pollutant shall include but not be limited to any Pollutant identified pursuant to Section 307(a) of the Act.

3)	Any Wastes or Wastewaters having a pH lower than 6.0 or in excess of 12 or having any other corrosive property capable of causing damage or hazard to the POTW, its equipment and/or its Personnel.

4) 	Any Wastes or Wastewaters solid or viscous substances in quantities or of such size capable of causing an obstruction to flow in the District Facilities or otherwise interfering with the proper operation of the POTW including as but not limited to, ash, ashes, cinders, sand, mud, straw, shavings, metal, glass, rags, feathers, tar, plastics, wood, un-ground garbage, whole blood, paunch manure, hair and fleshings, entrails, grease, animal guts or tissues, bones, spent lime, beer or distillery slops, grain processing waste, hops, stone or marble dust, grass clippings, waste paper, gas, asphalt residues from refining, processing of fuel or lubricating oil, glass grinding, polishing wastes, paper dishes, cups, milk containers, etc., either whole or ground by garbage grinders, and all other solid objects, materials, refuse and debris not normally contained in Sanitary Sewage.

5) 	Wastewaters having a temperature greater than 140 degrees F (60 degrees C), or which will inhibit any wastewater biological activity or cause damage in the POTW resulting in Interference, but in no case Wastewaters which cause the temperature at the introduction into the POTW to exceed 40 degrees Centigrade (104 degrees Fahrenheit).

6) 	Any Wastes or Wastewaters containing fats, wax, grease or oils of animal or vegetable origin, whether emulsified or not, in excess of one hundred (100) mg/l or containing substances which may solidify or become viscous at temperatures between thirty-two (32) and one hundred forty (140) ° F (0 and 60°C) or in amounts that will cause obstruction of the flow in the POTW resulting in Interference.

7)	Any garbage that has not been properly shredded.  The installation and operation of any garbage grinder equipped with a motor of three-fourths (3/4) horsepower (0.76 hp metric) or greater shall be subject to the review and approval of the District.

8)	Any Wastes or Wastewaters containing strong acid iron pickling wastes or concentrated plating solutions, whether neutralized or not.

9)	Any Wastes or Wastewaters with objectionable color not removed in the treatment process, such as, but not limited to, dye wastes, and vegetable tanning solutions.

10)	Any Wastes or Wastewaters containing iron chromium, copper, zinc and similar objectionable or toxic substances; or wastes exerting an excessive chlorine requirement, to such a degree that any such material received in the composite sewage at the POTW exceeds the limits established by the District in compliance with applicable State and Federal regulations for such materials.

11)	Any noxious or malodorous liquids, gases, or solids which either singly or by interaction with other Wastes or Wastewaters are sufficient to create public nuisance or hazard to life or are sufficient to prevent entry into the sewers for maintenance and repair.

12)	Any Wastes or Wastewaters containing phenols or other taste or odor producing substances in such concentrations exceeding limits which may be established by the District, after treatment of the composite sewage, to meet the requirements of the State, Federal or other public agencies or jurisdiction for such discharge to the receiving waters.

13)	Any Wastes or Wastewaters containing radioactive wastes or isotopes of such half-life or concentration as may exceed limits established by the District in compliance with applicable State and Federal regulations.

14)	Any Wastes or Wastewaters containing material identified as hazardous waste according to 40 CFR Part 261.

15)	Any quantities of flow, concentrations, or both that constitute a "slug" as defined in Article 1 of these Pretreatment Rules and Regulations.

16)	Any Wastes or Wastewaters containing petroleum oil, non-biodegradable cutting oils or products of mineral oil origin, in amounts that will cause Interference or Pass Through.

17)	Any medical wastes such as but not limited to:  isolation wastes, infectious agents, human blood and blood products, pathological wastes, sharps, body parts, surgical wastes, potentially contaminated laboratory wastes, chemotherapy wastes and/or dialysis wastes.

18)	Any Wastes or Wastewaters containing substances which are not amenable to treatment or reduction by the sewage treatment process employed or are amenable to Treatment only to such degree that the POTW effluent cannot meet the requirements of other agencies having jurisdiction over Discharges to the receiving waters.

19)	Any substance which will cause the District’s wastewater treatment facilities to violate its MEPDES Permit and/or the receiving water quality standards.

20)	Any substances which may cause the District’s POTW effluent or any other product of the District Facilities such as residues, sludge, or scum, to be unsuitable for reclamation and reuse or to interfere with the reclamation process. In no case, shall a substance discharged to the District’s POTW cause the District Facilities to be in non-compliance with sludge use or disposal criteria, guidelines or regulations developed under Section 405 of the Act; any criteria, guidelines, or regulations affecting sludge used or disposal developed pursuant to the Solid Waste Disposal Act, the Clean Air Act, the Toxic Substances Control Act, or State Criteria applicable to the sludge management method being used.

21)	Household hazardous wastes including but not limited to paints, stains, thinners, pesticides, herbicides, anti-freeze, and battery acid.

22)	Storm water, surface water, ground water, artesian well water, roof runoff, subsurface drainage, condensate, deionized water, non-contact cooling water, or otherwise unpolluted Wastewaters.

23)	Detergents, surface-active agents, or other substances that may cause excessive foaming in the POTW.

24)	Any Wastewaters which cause a hazard to human life or create a public nuisance.

The limits fixed herein may be used as a guide in design and plant control but may be altered by the District as required or authorized by State or Federal law in the event of a cumulative overload on a particular drainage basin or wastewater treatment plant.

Section 2. Bypass

Bypass means the intentional diversion of waste streams from any portion of a User’s treatment facility.

Severe property damage means substantial physical damage to property, damage to the treatment facilities which causes them to become inoperable, or substantial and permanent loss of natural resources which can reasonably be expected to occur in the absence of a bypass.  Severe property damage does not mean economic loss caused by delays in production.

A.	A User may allow any bypass to occur which does not cause Pretreatment Standards or Requirements to be violated, but only if it is for essential maintenance to assure efficient operation.  These bypasses are not subject to the provision of paragraphs (B) and (C) below.

B.	Bypass Notifications

1).	If a User knows in advance of the need for a bypass, it shall submit prior notice to the District, at least ten (10) days before the date of the bypass, if possible.

2).	A User shall submit oral notice to the District of an unanticipated bypass that exceeds applicable Pretreatment Standards within twenty-four (24) hours from the time it becomes aware of the bypass.  A written submission shall also be provided within five (5) days of the time the User becomes aware of the bypass.  The written submission shall contain a description of the bypass and its cause; the duration of the bypass, including exact dates and times, and, if the bypass has not been corrected, the anticipated time it is expected to continue; and steps taken or planned to reduce, eliminate, and prevent reoccurrence of the bypass.   The District may waive the written report on a case-by-case basis if the oral report has been received within twenty-four (24) hours.

3).		A bypass is prohibited, and the District may take an enforcement action against a User for a bypass, unless

(a) Bypass was unavoidable to prevent loss of life, personal injury, or severe property damage;

(b) There were no feasible alternatives to the bypass, such as the use of auxiliary treatment facilities, retention of untreated wastes, or maintenance during normal periods of equipment downtime.  This condition is not satisfied if adequate back-up equipment should have been installed in the exercise of reasonable engineering judgment to prevent a bypass which occurred during normal periods of equipment downtime or preventive maintenance; and

(c) The User submitted notices as required under paragraph (B) of this above.

4).   	The District may approve an anticipated bypass, after considering its adverse effects, if the District determines that it will meet the three conditions listed in paragraph (3)(a) above.

Article III – Rates, Service Charges

The Sanford Sewerage District rates are set by the Board of Trustees and are available through the Superintendent at the District office, 281 River St. Springvale Maine 04083.

Article IV – Commercial and Industrial Discharge to Public Sewers Program

Section 1 – Pretreatment Purpose and Policy

The purpose and policy of the “Commercial and Industrial Discharge to Public Sewers " is to set forth uniform requirements for direct and indirect contributors into the wastewater collection and treatment system of the Sanford Sewer District (SSD), enabling the District to comply with all applicable State laws (06-096 CMR Chapter 528 Pretreatment Program) and Federal laws required by the Clean Water Act of 1972 and the General Pretreatment Regulations (40 CFR, Part 403).

This Article provides for the regulation of direct and indirect contributors to the District’s wastewater system through the issuance of permits to certain non-domestic Dischargers and through enforcement of general requirements for the other Dischargers, authorizes monitoring and enforcement activities, requires User reporting, establishes administrative review procedures, assumes that existing customer capacity will not be preempted, and provides for the setting of fees for the equitable distribution of costs resulting from the program established herein.   This Article shall apply to Users of the District Facilities. Any User that discharges wastewater into the District’s wastewater system, shall comply with the terms and conditions established in these Rules and Regulations, as well as any permits or orders issued hereunder.

Except as otherwise provided herein, the District shall administer, implement, and enforce the provisions of this Article.

The objective of these Rules and Regulations is to;

- Prevent the introduction of pollutants into the District’s wastewater system that will interfere with the operation of the system or negatively affect the quality of the resulting sludge or its marketability.

- Prevent the introduction of Pollutants into the District Facilities which will pass through the POTW, inadequately treated, into receiving waters or the atmosphere or otherwise be incompatible with the POTW.

- Provide for the equitable distribution of the cost of the District Facilities

- Protect, in addition to the general public, the District’s Personnel who may come into contact with sewage, sludge and effluent in the course of their employment.

- Ensure the District’s compliance with its MEPDES permit conditions, sludge use and disposal requirements and any other Federal or State laws to which the District Facilities may be subject.

- Improve the opportunity to recycle and reclaim wastewater and sludges from the system.

Section 2. - Administration

District Actions.  If any waters or wastes are discharged or are proposed to be discharged to the Public Sewers, which contain the substances or possess the characteristics enumerated in this Article, and which in the judgment of the District may have a deleterious effect upon the District Facilities, processes, equipment or receiving waters, which otherwise create a hazard to life or constitute a public nuisance, the District may:

1)	 Reject the Wastes;

2)	 Require pretreatment to an acceptable condition for discharge to the Public Sewers;

3)	Require control over the quantities and rates of Discharge; and/or

4)	Require payment to cover the added cost of handling and treating the Wastes or Wastewaters not covered by existing sewer charges under the provisions of these Rules and Regulations.

Section 3. - District Review of Pretreatment Facility Plans

If the District permits the installation of pretreatment or equalization of waste flows, the design and installation of the plants and equipment shall be subject to the review and approval of the District and subject to the requirements of all applicable codes, ordinances and laws.

Detailed plans describing such facilities and operating procedures shall be submitted to the District for review and shall be accepted by the District before such facilities are constructed or operating procedures are initiated.  The User shall reimburse the District for any expense, including the cost for the District to obtain expert advice or assistance in order to review and to accept the facilities and operating procedures.  The review of such plans and operating procedures shall in no way relieve the User from the responsibility of modifying such facilities as necessary to produce a discharge acceptable to the District under the provisions of the Rules and Regulations.  Any subsequent changes in the facilities or operating procedures shall be reported to and must be approved by the District prior to the User’s initiation of the changes.

Section 4. - Prohibited Wastes and Wastewaters

Prohibited Wastes and Wastewaters shall be processed or stored in such a manner that these wastes shall not be discharged to the District Facilities. All floor drains located in process or materials storage areas must discharge to the User’s pretreatment facility before connecting with the District Facilities.  When the District determines that a User(s) is discharging to the District Facilities, any substance(s) in such amounts as to interfere with the operation of the District Facilities, the District shall:

  - Advise the User(s) of the impact of the discharge on the District Facilities; and

  - Develop effluent limitation(s) for such User to correct the Interference with the District Facilities if none exist in these Rules and Regulations.

Section 5 – Fats, Oils and Grease (FOG)

Fats, Oils, and Grease Permit.  The District shall issue a Fats, Oils, and Grease (FOG) Permit to the approved applicants. Approved FOG Permits are required to discharge wastewater into the District Facilities for all Food Service Establishments, as defined in these Pretreatment Rules and Regulations.  Permits must be renewed annually.

The FOG Permit shall determine the required cleaning schedule of the Interceptor based on the FSE’s Menu Type, Number of Meals per Day, and the Size of the Installed Interceptor.

Any User planning expansion, remodeling, or process modifications must submit an updated FOG Permit Application, 30 days prior to starting the modifications. Upon approval, the District shall issue a revised FOG permit to the establishment.

Grease Interceptors. Grease Interceptors must be installed where food is prepared or dispenses, including but not limited to: restaurants, cafés, lunch counters, cafeterias, bars and clubs, hotels, bed-and-breakfasts, hospitals, ice cream parlors, sanitariums, factories, school kitchens, commercial kitchens, concession stands and nursing homes.

A Grease Interceptor is not required for individual dwelling units or for any single-family private living quarters. Establishments which are not connected to the District Facilities are exempt from the requirements of this Section as long as their waste is disposed of outside the jurisdiction of the Sanford Sewerage District.

Oil and sand interceptors shall be provided when, in the opinion of the District, they are necessary for the proper handling of liquid wastes containing any flammable wastes, sand or other harmful ingredients, except that such interceptors shall not be required for private living quarters or dwelling units.

All costs incidental but not limited to the application, permitting, installation, testing, inspection, and maintenance of the grease removal system shall be borne by the owner.

No wastewater, other than from kitchen fixtures or food processing equipment, shall be discharged into the Grease Interceptor unless approved by the District in writing.  Wastewater from sanitary sources such as toilets and bathroom sinks are prohibited from being discharged to the grease removal system.

Any User required to install a Grease Interceptor must size, select, install and properly maintain an approved type of grease interceptor in accordance with the District’s Grease Interceptor Sizing and Selection Method and FOG Permit Application.

An automatic electrical/mechanical Grease Interceptor or Grease Trap may be approved solely at the District’s discretion where physical constraints prevent the installation of an approved type of Grease Interceptor. These units shall be of substantial construction, water tight and equipped with easily removable covers which when secured in place shall be gastight and watertight. Internal units shall have a minimum capacity of 20 gallons per minute flow.

Sand and oil interceptors, when in the opinion of the District are necessary, shall conform to the standards described in the current Uniform Plumbing Code.

All Grease Interceptors shall be approved by the District. Hydromechanical Grease Interceptors shall comply with ASME A112.14.3, PDI G101, or CSA B481. When approved for installation, Gravity Grease Interceptors shall comply with IAPMO/ANSI Z1001, and Automatic Grease Removal Units shall comply with ASME A112.14.4 or CSA B481. Any plans for such grease removal systems as required in this Section shall be submitted for District review and approval prior to installation. The District shall be notified seventy-two (72) hours prior to installation of any Grease Interceptor in order to inspect and oversee the installation.

All grease interceptors shall be installed with all of the manufacturer’s required components for the unit to operate as designed. These components are required to remain installed at all times in order for the device to be compliant.

All Grease Interceptors shall be made from materials that are compatible with a pH of 3. Grease Interceptors that are made from materials that are subject to corrosion such as concrete or steel, shall be lined or coated with a durable material approved by the District that is compatible with a pH of 3 and that cannot be easily penetrated, scraped off or removed. Acid Resistant Enamel coatings (A.R.E.) are not allowed.

All wastewater from food preparation operations and/or washing and clean-up operations, including, but not limited to, pot sinks, pre-rinse stations, hand sinks, work stations, soup kettles, braising pans, wok ranges, mop sinks, floor sinks, floor drains, emergency floor drains and wastewater generated from exhaust fan hood cleaning operations, or any other fixtures required by the District, must discharge to the Grease Interceptor. All automatic dishwasher wastewater must bypass the Grease Interceptor and be discharged directly into the Sewer System.

Garbage disposal units are not permitted unless approved by the District.

Each Grease Interceptor shall be so located as to be readily and easily accessible for cleaning, inspection, and/or sampling at all times.  A suitable sampling location shall be provided for sampling of the discharges from Grease Interceptors. All automatic Grease Removal Devices must have a sampling valve installed on the discharge piping with a minimum clearance of eight (8) inches for the installation of sampling bottles, or an in-ground sample well on the discharge side.

FSE properties in which Grease Interceptors are installed must, on Grease Cleaning Record Forms provided by the District, maintain a record of the dates that such Grease Interceptors are cleaned as required by the FSE’s FOG permit. These forms remain the property of the District and are recognized proof of cleaning, and will be collected by the District at the end of each calendar year. Seasonal businesses are exempt from cleaning requirements during the times of year when they are closed.

Grease Cleaning Record Forms shall be displayed proximal to the location of the Grease Interceptor if applicable. At a minimum said forms will be kept in prominent location, accessible to the District at all times.

When approved for installation, Gravity Grease Interceptors shall be inspected monthly by the FSE and shall be cleaned by a State licensed liquid waste hauler whenever the level of grease is 25% of the effective depth of the trap or at least every three months, whichever is sooner or more frequently if required. The FSE shall be responsible for the proper disposal by legal and environmentally safe methods of the captured material and shall maintain records of inspections and the date, name of hauler, quantity hauled and means of disposal on the Record Form provided by the District.

When approved for installation, Automatic Grease Removal Devices shall be cleaned as needed and at the minimum of one time every four weeks or more frequently if required. Automatic Grease Removal Devices shall be cleaned by a State licensed liquid waste hauler at least once per year.

Hydromechanical Grease Interceptors shall be cleaned by a State licensed liquid waste hauler at the frequency determined on the District’s FOG Permit Application or more or less often as determined by the District.

When approved by the District, self-cleaning by FSE’s must include completely removing the interceptor’s contents, all internal surfaces scrubbed and cleaned to remove accumulated FOG, the interceptor refilled with fresh water, and the evacuated contents properly disposed of.

No user may cause or allow to be introduced into a Grease Interceptor any chemical, bacterial, enzyme or other additive, directly or indirectly, that interferes with the normal operation of the device or causes pass-through of prohibited substances.

If inspection and/or sampling indicates minimum cleaning requirements are not adequate, additional cleaning shall be required.

Inspections.  As outlined in these Pretreatment Rules and Regulations any FSE with a FOG Permit shall grant to the District permission to conduct unannounced inspections of any installed Interceptor(s). Should an inspection show a violation of this section, the FSE shall be subject to the enforcement as described in these Pretreatment Rules and Regulations.

Section 6. - National Categorical Pretreatment Standards

Users must comply with the categorical Pretreatment Standards found at 40 CFR Chapter I, Subchapter N, Parts 405–471.

A.	Where a categorical Pretreatment Standard is expressed only in terms of either the mass or the concentration of a pollutant in wastewater, the Superintendent may impose equivalent concentration or mass limits in accordance with Paragraphs E and F of this Section.

B.	When the limits in a categorical Pretreatment Standard are expressed only in terms of mass of pollutant per unit of production, the Superintendent may convert the limits to equivalent limitations expressed either as mass of pollutant discharged per day or effluent concentration for purposes of calculating effluent limitations applicable to individual Industrial Users.

C.  	The Superintendent may convert the mass limits of the categorical Pretreatment Standards of 40 CFR Parts 414, 419, and 455 to concentration limits for purposes of calculating limitations applicable to individual Industrial Users.  The conversion is at the discretion of the Superintendent.

D. 	Once included in its permit, the Industrial User must comply with the equivalent limitations developed in this Section in lieu of the promulgated categorical Standards from which the equivalent limitations were derived.

E.  	Many categorical Pretreatment Standards specify one limit for calculating maximum daily discharge limitations and a second limit for calculating maximum Monthly Average, or 4-day average, limitations. Where such Standards are being applied, the same production or flow figure shall be used in calculating both the average and the maximum equivalent limitation.

Section 7 – Pretreatment Facilities.

Users shall provide necessary Wastewater treatment as required to comply with this Article and shall achieve compliance with all Categorical Pretreatment Standards, local discharge limits, and prohibitions within the time limitations specified by the District. Any facilities required to pretreat wastewater to a level acceptable to the District shall be provided, operated, and maintained at the User’s expense. Detailed plans showing the pretreatment facilities and operating procedures shall be submitted to the District for review and shall be approved prior to construction of the facilities.  The review of such plans and operating procedures will in no way relieve the User from the responsibility of modifying the facility as required to produce an effluent which complies with the provisions of these Rules and Regulations or from liability for non-compliance with Pretreatment Standards or Pre-treatment Requirements and these Rules and Regulations.  Any subsequent changes in the pretreatment facilities or method of operation shall be reported to and be approved by the District prior to the User’s initiation of the changes.

a.	Whenever deemed necessary, the District may require Users to restrict their Discharge during peak flow periods, designate that certain Waste specific sewers, relocate and or consolidate points of discharge, separate other sewage wastewaters and waste streams from Industrial waste streams, and such other conditions as may be required to protect the POTW and determine the User’s compliance with the requirements of these Rules and Regulations.

b.	The District may require any User discharging into the POTW to install and maintain on their property and at their expense, a suitable storage and flow-control facility to ensure equalization of flow.  An individual wastewater discharge permit may be required solely for flow isolation.

c.	Grease, oil, and sand interceptors shall be provided when, in the opinion of the District, they are necessary for the proper handling of wastewater containing excessive amounts of grease, flammable substance, sand, or other harmful substances; except that such interceptors shall not be required for residential Users.  All interceptor units shall be of a type and capacity approved by the District and shall be so located to be easily accessible for cleaning and inspection.  Such interceptors shall be inspected, cleaned, and repaired regularly, as needed by the FSE at his/her expense.

Section 8. - State Pretreatment Standards

Users must comply with State of Maine Pretreatment Standards (CMR 06-096 Chapter 528 Pretreatment Program)

Section 9. – Records and Public Notification Procedures

All records relating to compliance with these Rules and Regulations, applicable Pretreatment Standards and Pretreatment Requirements as defined in 40 CFR §403.3(j) and (r) shall be made available to officials of the EPA, DEP, or District upon request.  In addition, pursuant to the public participation requirements of 40 CFR Part 25, the District shall publish annually, in a newspaper of general circulation that provides meaningful public notice within the District, a list of the Users which, at any time during the previous twelve (12) months, were in Significant Noncompliance with these Rules and Regulations, applicable Pretreatment Standards and Requirements. The term Significant Noncompliance shall be applicable to all Commercial/Industrial Users (or any other User that violates these Rules and Regulations and shall mean:

(a)	Chronic violations of wastewater discharge limits, defined here as those in which sixty-six percent (66%) or more of all measurements taken for the same pollutant parameter taken during a six (6) month period exceed (by any magnitude) a numeric Pretreatment Standard or Requirement, including Instantaneous Limits;

(b)	Technical Review Criteria (TRC) violations, defined here as those in which thirty-three percent (33%) or more of wastewater measurements taken for each pollutant parameter during a six (6) month period equals or exceeds the product of the numeric Pretreatment Standard or Requirement including Instantaneous Limits, multiplied by the applicable criteria (1.4 for BOD, TSS, fats, oils and grease, and 1.2 for all other pollutants except pH);

(c)	Any other violation of a Pretreatment Standard or Requirement (Daily Maximum, long-term average, Instantaneous Limit, or narrative standard) that the District determines has caused, alone or in combination with other discharges, Interference or Pass Through, including endangering the health of POTW Personnel or the general public;

(d)	Any discharge of a Pollutant that has caused imminent endangerment to the public or to the environment, or has resulted in the District’s exercise of its emergency authority to halt or prevent such a discharge;

(e)	Failure to meet, within ninety (90) days of the scheduled date, a compliance schedule milestone contained in an individual wastewater discharge permit or enforcement order for starting construction, completing construction, or attaining final compliance;

(f)	Failure to provide within forty-five (45) days after the due date, any required reports, including baseline monitoring reports, periodic self-monitoring reports, and reports on compliance with compliance schedules;

(g)	Failure to accurately report noncompliance; or

(h)	Any other violation(s), which may include a violation of Best Management Practices, which the District determines will adversely affect the operation or implementation of these Rules and Regulations.

Section 10. – District’s Right of Revision

The District reserves the right to establish by revision or amendment of these Rules and Regulations more stringent limitations or requirements on discharges to the wastewater disposal system if deemed necessary to comply with the objectives presented in this Article.

Section 11.  - Excessive Discharge

No User shall ever increase the use of process water or, in any way, attempt to dilute a discharge as a partial or complete substitute for adequate treatment to achieve compliance with the limitations contained in the Federal Categorical Pretreatment Standards, or in any other pollutant-specific limitation developed by the District or State.

Section 12. – Discharges

Each User shall provide protection from discharge of prohibited materials or other substances regulated by these Rules and Regulations. Facilities to prevent accidental discharge of prohibited materials shall be provided and maintained at the owners or Users own cost and expense. Detailed plans showing facilities and operating procedures to provide this protection shall be submitted to the District for review and shall be approved before construction of the facility. Review and approval of such plans and operating procedures shall not relieve the User from the responsibility to modify the Users facility as necessary to meet the requirements of this section. In the case of a discharge, it is the responsibility of the User to:

A.	Immediate Notice – Regardless of time of day, the User shall immediately telephone and notify the District of the incident. The notification shall include location of discharge, type of waste, concentration and volume, and corrective actions.

B.	Written Notice - Within five (5) days following an accidental discharge; the User shall submit to the District a detailed written report describing the cause of the discharge and the measures to be taken by the User to prevent similar future occurrences. Such notification shall not relieve the User of any expense, loss, damage, or other liability which may be incurred as a result of damage to the District Facilities, fish kills, or any other damage to Person or property; nor shall such notification relieve the User of any fines, civil charges, or other liability which may be imposed by this article or other applicable law.

C.	Notice to Employees - A notice shall be permanently posted on the Users bulletin board or other prominent place advising employees whom to call in the event of a Discharge. Employers shall insure and document that all employees who may cause or suffer such a Discharge to occur are advised of the emergency notification procedure.

Section 13. -   Fees

A. 	Purpose

The fees established herein are to provide for the recovery of the costs associated with the implementation and maintenance of the Pretreatment Program from Users of the District Facilities.

B.	Charges and Fees

The District may adopt charges and fees that may include:

- Fees for reimbursement of costs of setting up and operating the District’s Pretreatment Program;

- Fees for monitoring, inspections and compliance procedures;

- Fees for reviewing discharge procedures and construction;

- Fees for filing appeals;

- Fees for consistent removal (by the District) of pollutants otherwise subject to Federal Pretreatment Standards;

- Other fees as the District may deem necessary to carry out the requirements contained herein;

- Penalties for violation of pretreatment/discharge requirements.

- Fees for the direct impacts of the User’s discharge on District systems.

These fees relate solely to the matters covered by this section and are separate from all other fees chargeable by the District.

Section 14. Pretreatment Procedures (Permitting, Compliance Tracking, and Enforcement)

The District’s Pretreatment Program consists of four major elements: the identification and categorization of Dischargers and determination of pretreatment requirements; the issuance of Permits; the tracking of Dischargers to ensure compliance with Permits; and the enforcement of all rules and regulations. The intent of these procedures is to establish the administrative mechanism to allow the efficient and effective implementation of this section.

A.  	Identification and Categorization of Dischargers and Determination of Pretreatment

Requirements

1)	 Identification of Dischargers

All Dischargers of Non-domestic Waste are subject to the pretreatment regulations of this Article and must be reviewed to evaluate applicability of pertinent requirements.

a) Existing Dischargers - The primary tool for identifying existing Dischargers who may be subject to the District’s pretreatment requirements is a preliminary screening questionnaire and accompanying letter of transmittal. The questionnaire and letter will be sent to any known or suspected Discharger who has not been categorized as below. The function of the questionnaire is to obtain sufficient information to determine what further action will be required to categorize the Discharger.  Should there be no response from the recipient to the questionnaire, a second questionnaire will be sent with a letter of transmittal detailing the eventual consequence of non-response. Should the second questionnaire elicit no response, the name of the Discharger will be referred for follow-up under enforcement procedures.  A record of all businesses contacted, and the subsequent responses will be maintained.

b) New Dischargers - The District will provide each applicant for a business license a questionnaire that will accompany the sewer permit application. The applicant for the business license shall complete the questionnaire and submit it with the sewer permit application to the District.  After the completed questionnaire is returned, and the responses provided evaluated by the District, the business will be placed in one of the four categories of the Pretreatment program as described in paragraph 2 below.

2)  	Categorization

Upon receipt of the required information, each Discharger will be placed into one of the following categories for program management. Categorization will be made in as fair and equitable manner as possible. General guidelines for identifying category placement are given below.

Commercial - Those businesses which have no discharge other than normal sanitary wastewater, or whose non-sanitary discharge has no significant effect on the District Facilities shall be placed in this category. Businesses so designated may be tracked by the District. These Businesses will be maintained in the District’s inventory in case a change in status is required in the future. For those businesses designated Commercial, a permit may be executed between the District and Discharger.

Food Service Establishments - Businesses that prepare and/or serve food commercially are placed in this category. A business so categorized will be monitored for oil/grease and any other constituents that, in the District’s judgment, may interfere with the operation and maintenance of the District Facilities. In addition, any discharger placed in this category will be required to fill out a FOG (Fats, Oil, & Grease) Permit Application.  A permit delineating specific requirements for the discharger will be executed between the District and the discharger so designated.  In addition, the Discharger will be subject to the District’s compliance tracking program.

Industrial User - Businesses with Wastewater Discharges that do not fall under State or Federal Industrial pretreatment guidelines. Businesses which contain Discharges that contain some constituent of concern to the District, or whose Discharge may interfere with the operation and maintenance of the District Facilities will be subject to control by the District. A permit delineating specific requirements for the Discharger will be executed between the District and the Discharger for all Dischargers designated Industrial. In addition, the Discharger will be subject to the District’s compliance tracking program.

Significant Industrial User - Businesses that are subject to State and Federal Industrial Pretreatment rules and regulations will be placed in this category. Generally, in accordance with State requirements, a business which is discharging a wastewater with one or more of the following characteristics will be placed in this Category:

- An industrial user subject to Categorical Pretreatment Standards under 40CFR (Code of Federal Regulations), 403.6 and 40 CFR, Chapter I, subchapter N and 06-096 CMR Chapter 528 Pretreatment Program. Any industry that falls under these categories is considered a SIU whether it has process discharge to the sewer or not. These industries are Categorical Industrial Users (CIUs);

- Any industry which discharges an average of 25,000 gallons per day or more of process wastewater to the sewer system (excluding sanitary, contact cooling and boiler blowdown wastewater);

Businesses placed in this category will be required to execute a permit with the District for the purpose of providing the District a means of regulating the Discharge and will be subject to the District’s compliance tracking program.

3)  	Monitoring Facilities

The District shall require all Food Service Establishments, Industrial Users, and Significant Industrial Users to provide and operate, at the User’s own expense, monitoring facilities to allow inspection, sampling, and flow measurement of the Building Sewer and/or internal drainage system. The monitoring facility shall normally be situated in the User’s premises, but the District may, when such a location would be impractical or cause undue hardship on the User, allow the facility to be constructed in the public street or sidewalk area with the required, appropriate permission from the City and located so that it will not be obstructed by landscaping or parked vehicles. There shall be ample room in or near such sampling manhole or facility to allow accurate sampling and preparation of samples for analysis. The facility, sampling, and measuring equipment shall always be maintained in a safe and proper operating condition at the expense of the User.

4)  	Inspection and Sampling

The District shall inspect the facilities of any User to ascertain whether the User is complying with all requirements and the purpose of this section is being met. Persons or occupants of premises where wastewater is created or discharged shall allow the District or its representative ready access at all reasonable times to all parts of the premises for the purposes of inspection, sampling, records examination or in the performance of any of their duties. The District and the Approval Authority shall have the right to set up on the Users property such devices as are necessary to conduct sampling inspection, compliance monitoring and/or metering operations. Where a User has security measures in force which would require proper identification and clearance before entry into their premises, the User shall make necessary arrangements with their security guards so that upon presentation of suitable identification, Personnel from the District, MDEP, and EPA will be permitted to enter, without delay, for the purposes of performing their specific responsibilities.

5)  	Determination of Pretreatment Requirements

After the User is placed in one of the categories previously described, requirements for the User must be determined. The District will make Food Service Establishments and Industrial User requirements based on available information and Significant Industrial Users in coordination with MDEP. Effluent limits for priority/categorical pollutants will be in accordance with those promulgated by EPA and MDEP; unless more stringent limits are necessary to protect the District’s wastewater collection and treatment system.

- Modification of Program Requirements for Dischargers

Periodically, changes in pretreatment requirements of existing dischargers may be appropriate. When such changes are deemed necessary, the procedures listed below are applicable:

a. 	The Dischargers will be notified in writing of the proposed change and of the basis for the change.

b. 	Included in the notice of change will be any draft permit or requirements, if appropriate.

c. 	The proposed change in discharger requirements will be effective thirty (30) days after notice. Should a discharger object to the change, such objection must be registered with the District within thirty (30) days of receipt of the notice of proposed change.

d.  	The filing of a request by the User for a permit modification does not stay any condition of its existing permit.

Section 15. - Accidental Discharge/Slug Discharge Control Plans

The District shall evaluate whether each SIU needs an accidental discharge/slug discharge control plan or other action to control Slug Discharges.  The District may require any User to develop, submit for approval, and implement such a plan or take such other action that may be necessary to control Slug Discharges.  Alternatively, the District may develop such a plan for any User.  An accidental discharge/slug discharge control plan shall address, at a minimum, the following:

- A.	Description of discharge practices, including nonroutine batch discharges;

B.	Description of stored chemicals;

C.	Procedures for immediately notifying the District of any accidental or Slug Discharge, as required by Section 6.6 of these Rules and Regulations; and

D.	Procedures to prevent adverse impact from any accidental or Slug Discharge.  Such procedures include, but are not limited to, inspection and maintenance of storage areas, handling and transfer of materials, loading and unloading operations, control of plant site runoff, worker training, building of containment structures or equipment, measures for containing toxic organic pollutants, including solvents, and/or measures and equipment for emergency response.

Section 16. -  	Hauled Wastewater

A.	Septic tank waste may be introduced into the POTW only at locations designated by the Superintendent, and at such times as are established by the Superintendent.  Such waste shall not violate these Rules and Regulations or any other requirements established by the Sanford Sewerage District. The Superintendent may require septic tank waste haulers to obtain individual wastewater discharge permits.

B.	The District  may require haulers of industrial waste to obtain individual wastewater discharge permits.   The District may require generators of hauled industrial waste to obtain individual wastewater discharge permits.   The District also may prohibit the disposal of hauled industrial waste.  The discharge of hauled industrial waste is subject to all other requirements of these Rules and Regulations.

C.	Industrial waste haulers may discharge loads only at locations designated by the District.  No load may be discharged without prior consent of the District. The District may collect samples of each hauled load to ensure compliance with applicable Standards.   The District may require the industrial waste hauler to provide a waste analysis of any load prior to discharge.

D.	Industrial waste haulers must provide a waste-tracking form for every load.  This form shall include, at a minimum, the name and address of the industrial waste hauler, permit number, truck identification, names and addresses of sources of waste, and volume and characteristics of waste.  The form shall identify the type of industry, known or suspected waste constituents, and whether any wastes are RCRA hazardous wastes.

Section 17. - Regulation of Waste Received from Other Jurisdictions

The Sanford Sewerage District will only accept industrial wastewater from the City of Sanford’s territory. Domestic Septic Wastes are accepted from communities in York County, Maine.

Section 18. - Permits

The basis for regulating discharges to the District Facilities under this Article will be through permits between the Discharger and the District. Permits will be issued and enforced by the District in coordination with the MDEP (when required) but are directly controlled by the District. These permits will specifically identify all pretreatment requirements to be enforced by the District that the Discharger must meet and will provide the District that authority required by Federal pretreatment regulations. Dischargers may be subject to other State and Federal pretreatment requirements not included in the District’s permit.  Should a User be required to construct a pretreatment facility, such a requirement and a schedule for completion of such facility will be included as an enforceable portion of the User’s Permit.

A.	Permit Application

Proposed new Users which are subject to Pretreatment Requirements under this Article shall apply at least 45 days prior to connecting to or contributing to the District Facilities. In support of the application, the User shall submit, in units and terms appropriate for evaluation. The District will evaluate the data furnished by the User and may require additional information. After evaluation and acceptance of the data furnished, the District may issue a Pretreatment Wastewater Discharge Permit subject to terms and conditions required by the District, which will be part of User’s Permit.

B.	Permit Conditions

Pretreatment Wastewater Discharge Permits shall be expressly subject to all provisions of this section and all other applicable regulations, user charges and fees established by the District, and may contain the following:

1.	The unit charge or schedule of User charges and fees for the wastewater to be discharged to a community sewer;

2.	Local Limits

The Superintendent is authorized to establish Local Limits pursuant to 40 CFR 403.5(c).

The following pollutant limits are established to protect against Pass Through and Interference.  No person shall discharge wastewater containing in excess of the following daily Maximum Concentrations.

2.19     	mg/l aluminum

34.8 		mg/l ammonia Nitrogen

0.008            mg/l arsenic, total (revised July 2013)

300.0	            mg/l BOD5

0.001	            mg/l cadmium, total

0.613	            mg/l chromium, total

0.318  	mg/l copper, total

0.110   	mg/l cyanide

0.00126	mg/l lead, total

0.08	            mg/l mercury, total

0.238		mg/l nickel, total

8.97		mg/I phosphorus, total

0.055		mg/l selenium, total

0.002  	mg/l silver, total

420.0		mg/l total suspended solids

0.173		mg/l zinc, total

6.0 to 12.0 S.U.     pH

The above limits apply at the point where the wastewater is discharged to the POTW.  All concentrations for metallic substances are for total metal unless indicated otherwise.   The Superintendent may impose mass limitations in addition to the concentration-based limitations above.

3.	Dilution. No User shall ever increase the use of process water, or in any way attempt to dilute a discharge, as a partial or complete substitute for adequate treatment to achieve compliance with a discharge limitation unless expressly authorized by an applicable Pretreatment Standard or Requirement.

4.	The District may develop Best Management Practices (BMPs), by Rules and Regulations or individual wastewater discharge permits, to implement Local Limits and the requirements of Article II.

C.	Permit Duration

Pretreatment Wastewater Discharge Permits shall be issued for a specified time period, not to exceed five (5) years. A permit may be issued for a period of less than a year or may state the specific date of expiration. A minimum of 180 days prior to the expiration of the User’s existing permit, the User shall apply for renewal of the permit. The terms and conditions of the permit may be subject to modification by the District during the term of the permit as limitations or requirements are modified or other just cause exists. The User shall be informed of any proposed changes in his permit at least 30 days prior to the effective date of the change. Any changes or new conditions in the permit shall include a time schedule for compliance.

D.	Permit Transfer

Individual wastewater discharge permits may be transferred to a new owner or operator only if the permittee gives at least 60 days advance notice to the District and the District approves the individual wastewater discharge permit transfer.  The notice to the District must include a written certification by the new owner or operator which:

1).	States that the new owner and/or operator has no immediate intent to change the facility’s operations and processes;

2).  	Identifies the specific date on which the transfer is to occur; and

3).	Acknowledges full responsibility for complying with the existing individual wastewater discharge permit.

Failure to provide advance notice of a transfer renders the individual wastewater discharge permit void as of the date of facility transfer.

E.	Confidential Information

Information and data on a User obtained from reports, questionnaires, permit applications, permits and monitoring programs and from inspections shall be available to the public or other governmental agency without restriction unless the User specifically requests and is able to demonstrate to the satisfaction of the District that the release of such information would divulge information, processes or methods of production entitled to protection as trade secrets of the User.

When so requested by the Person furnishing a report, the portions of a report which might disclose trade secrets or secret processes shall not be made available for inspection by the public but shall be made available upon written request to governmental agencies for uses related to the Permit, the MEPDES, and/or Pretreatment Programs; provided, however, that such portions of a report shall be available for use by the State or any state agency in judicial review or enforcement proceedings involving the Person furnishing the report. Wastewater constituents and characteristics will not be recognized as confidential information unless deemed by State law confidential.

The District shall not transmit to any governmental agency or to the general public information accepted by the District as confidential, until and unless a ten-day notification is given to the User.

F. 	Permit Modification

The District may modify an individual wastewater discharge permit for good cause, including, but not limited to, the following reasons:

1) 	To incorporate any new or revised Federal, State, or local Pretreatment

Standards or Requirements;

2) 	To address significant alterations or additions to the User’s operation,

processes, or wastewater volume or character since the time of the individual

wastewater discharge permit issuance;

3) 	A change in the POTW that requires either a temporary or permanent reduction or elimination of the authorized discharge;

4) 	Information indicating that the permitted discharge poses a threat to the Sanford Sewerage District’s POTW, Sanford Sewerage District’s personnel, or the receiving waters.

5) 	Violation of any terms or conditions of the individual wastewater discharge permit;

6) 	Misrepresentations or failure to fully disclose all relevant facts in the wastewater discharge permit application or in any required reporting;

7) 	Revision of or a grant of variance from categorical Pretreatment Standards pursuant to 40 CFR 403.13;

- 8) 	To correct typographical or other errors in the individual wastewater discharge permit; or

9) 	To reflect a transfer of the facility ownership or operation to a new owner or operator where requested in accordance with Paragraph D above.

G. 	Permit Revocation

The District may revoke an individual wastewater discharge permit for good cause, including, but not limited to, the following reasons:

1).	Failure to notify the District of significant changes to the wastewater prior to the changed discharge;

2).  	Failure to provide prior notification to the District of changed conditions;

3).  	Misrepresentation or failure to fully disclose all relevant facts in the wastewater discharge permit application;

4).  	Falsifying self-monitoring reports and certification statements;

5).  	Tampering with monitoring equipment;

6).  	Refusing to allow the District timely access to the facility premises and records;

7).  	Failure to meet effluent limitations;

8).  	Failure to pay fines;

- 9). 	Failure to pay sewer charges;

10).  	Failure to meet compliance schedules;

11).  	Failure to complete a wastewater survey or the wastewater discharge permit application;

12).  	Failure to provide advance notice of the transfer of business ownership of a permitted facility; or

13).	Violation of any Pretreatment Standard or Requirement, or any terms of the wastewater discharge permit or these Pretreatment Rules and Regulations.

Individual wastewater discharge permits shall be voidable upon cessation of operations or transfer of business ownership.  All individual wastewater discharge permits issued to a User are void upon the issuance of a new individual wastewater discharge permit to that User.

H.	Permit Reissuance

A User with an expiring individual wastewater discharge permit shall apply for individual wastewater discharge permit reissuance by submitting a complete permit application a minimum of 90 days prior to the expiration of the User’s existing individual wastewater discharge permit.

I.	Reporting Requirements

Baseline Monitoring Reports

1).	Within either one hundred eighty (180) days after the effective date of a categorical Pretreatment Standard, or the final administrative decision on a category determination under 40 CFR 403.6(a)(4), whichever is later, existing Categorical Industrial Users currently discharging to or scheduled to discharge to the POTW shall submit to the District a report which contains the information listed in paragraph 2, below.  At least ninety (90) days prior to commencement of their discharge, New Sources, and sources that become Categorical Industrial Users subsequent to the promulgation of an applicable categorical Standard, shall submit to the District a report which contains the information listed in paragraph 2, below.  A New Source shall report the method of pretreatment it intends to use to meet applicable categorical Standards.  A New Source also shall give estimates of its anticipated flow and quantity of pollutants to be discharged.

- 2).	Users described above shall submit the information set forth below:

Identifying Information.

a.	The name and address of the facility, including the name of the operator and owner.

b.	Contact information, description of activities, facilities, and plant production processes on the premises.

c.	Description of operations.

d.	Time and duration of discharges.

e.	The location for monitoring all wastes covered by the permit.

f.	Flow measurement.

g.	Measurement of pollutants.

h.	Any other information as may be deemed necessary by the District to evaluate the permit application.

3)	Compliance Certification.  A statement, reviewed by the User’s Authorized Representative and certified by a qualified professional, indicating whether Pretreatment Standards are being met on a consistent basis, and, if not, whether additional operation and maintenance (O&M) and/or additional pretreatment is required to meet the Pretreatment Standards and Requirements.

4)	Compliance Schedule.  If additional pretreatment and/or O&M will be required to meet the Pretreatment Standards, the shortest schedule by which the User will provide such additional pretreatment and/or O&M must be provided.  The completion date in this schedule shall not be later than the compliance date established for the applicable Pretreatment Standard.

5)	Signature and Report Certification.

Compliance Tracking

The purpose of the compliance tracking program is to insure that all Permitted Dischargers are meeting the terms of their Permits.  There are four components of the compliance tracking program.

1)	Self-Monitoring Reports

Significant Industrial Users are required to submit a self-monitoring report

semi-annually to the District as designated in their Permit. This report is to be submitted to the District so that it is received no later than the 14th day following the monitoring period covered. Failure to submit such report will be a breach of the Permit and could result in enforcement action.

2)	Industrial Users are required to submit a semi-annual self-monitoring report, as designated in their Permit with the District so that it is received no later than the 14th day following the monitoring period covered. Failure to submit such report will be a breach of the Permit and could result in enforcement action.

3)	Food Service Establishments are not required to submit a monthly, self-monitoring report. The district does all official monitoring.

J.	Compliance Evaluation Inspections

The purpose of compliance evaluation inspections (CEI) is to insure the proper operation of any pretreatment facilities specified in Permits with Permitted dischargers. These inspections are a "walk-through" type and do not involve effluent sampling. These inspections should confirm that all required facilities are in place and being properly operated. A CEI may be done concurrently with the compliance sampling inspection (CSI) described below. All Category permitted facilities will receive a CEI at least annually.

K.	Compliance Sampling Inspection

The purpose of the compliance sampling inspection (CSI) is to ensure that those effluent limits specified in a Discharger’s permit are being achieved.

a) During a CSI for permitted dischargers, samples will be taken from the specified location noted within the permit.

Any User that meets one or more of the following criteria shall be considered in significant noncompliance:

a) Chronic violations of wastewater discharge limits, defined here as those in which 66% or more of all the measurements taken during a 6-month period exceed (by any magnitude) the daily maximum limit or the average limit for the same parameter.

b) Technical Review Criteria (TRC) violations, defined here as those in which 33% or more of all the measurements for each pollutant parameter taken during a 6-month period equal or exceed the product of the daily maximum limit or the average limit multiplied by the applicable TRC (TRC=1.4 for BOD, TSS, fats, oil and grease, and 1.2 for all other parameters except pH).

c) Any other effluent violation of a pretreatment effluent limit (daily maximum or longer-term average) that the Control Authority determines has caused, alone or in combination with other discharges, interference or pass through (including endangering the health of York Sewer District Personnel or the general public).

d) Any discharge of a pollutant that has caused imminent endangerment to human health, welfare, or to the environment, or has resulted in the District exercising its emergency authority under paragraph 40 CFR Part 403.8(f)(l)(vi)(B) to halt or prevent such a discharge.

L.	Notice of Violation/Repeat Sampling and Reporting

If sampling performed by a User indicates a violation, the User must notify the District within twenty-four (24) hours of becoming aware of the violation.  The User shall also repeat the sampling and analysis and submit the results of the repeat analysis to the District within thirty (30) days after becoming aware of the violation. Resampling by the Industrial User is not required if the Sanford Sewerage District performs sampling at the User’s facility at least once a month, or if the Sanford Sewerage District performs sampling at the User between the time when the initial sampling was conducted and the time when the User or the Sanford Sewerage District receives the results of this sampling, or if the Sanford Sewerage District has performed the sampling and analysis in lieu of the Industrial User.

M.	Notification of the Discharge of Hazardous Waste

The Sanford Sewerage District prohibits the discharge of hazardous waste to its POTW.

This provision does not create a right to discharge any substance not otherwise permitted to be discharged by this ordinance, a permit issued thereunder, or any applicable Federal or State law.

N.	Analytical Requirements.

All pollutant analyses, including sampling techniques, to be submitted as part of a wastewater discharge permit application or report shall be performed in accordance with the techniques prescribed in 40 CFR Part 136 and amendments thereto, unless otherwise specified in an applicable categorical Pretreatment Standard.  If 40 CFR Part 136 does not contain sampling or analytical techniques for the pollutant in question, or where the EPA determines that the Part 136 sampling and analytical techniques are inappropriate for the pollutant in question, sampling and analyses shall be performed by using validated analytical methods or any other applicable sampling and analytical procedures, including procedures suggested by the  Superintendent or other parties approved by EPA.

O.	Sample Collection

Samples collected to satisfy reporting requirements must be based on data obtained through appropriate sampling and analysis performed during the period covered by the report, based on data that is representative of conditions occurring during the reporting period.

Except as indicated below, the User must collect wastewater samples using 24-hour flow-proportional composite sampling techniques, unless time-proportional composite sampling or grab sampling is authorized by the District.  Where time-proportional composite sampling or grab sampling is authorized by the Sanford Sewerage District the samples must be representative of the discharge.  Using protocols (including appropriate preservation) specified in 40 CFR Part 136 and appropriate EPA guidance, multiple grab samples collected during a 24-hour period may be composited prior to the analysis as follows: for cyanide, total phenols, and sulfides the samples may be composited in the laboratory or in the field; for volatile organics and oil and grease, the samples may be composited in the laboratory. Composite samples for other parameters unaffected by the compositing procedures as documented in approved EPA methodologies may be authorized by the Sanford Sewerage District as appropriate. In addition, grab samples may be required to show compliance with Instantaneous Limits.

Samples for oil and grease, temperature, pH, cyanide, total phenols, sulfides, and volatile organic compounds must be obtained using grab collection techniques.

For sampling required in support of baseline monitoring and 90-day compliance reports required in [40 CFR 403.12(b) and (d)], a minimum of four (4) grab samples must be used for pH, cyanide, total phenols, oil and grease, sulfide and volatile organic compounds for facilities for which historical sampling data do not exist; for facilities for which historical sampling data are available, the District may authorize a lower minimum. For the reports required by (40 CFR 403.12(e) and 403.12(h)), the Industrial User is required to collect the number of grab samples necessary to assess and assure compliance by with applicable Pretreatment Standards and Requirements.

P.	Inspection Summary Reports

Reports will be maintained of all inspection results by the District.

Section 19. – Reports of Changed Conditions

Each User must notify the District of any significant changes to the User’s operations or system which might alter the nature, quality, or volume of its wastewater at least 45 days before the change.

A.	The District may require the User to submit such information as may be deemed necessary to evaluate the changed condition, including the submission of a wastewater discharge permit application.

B.   	The District may issue a permit or modify an existing permit in response to changed conditions or anticipated changed conditions.

Section 20. – Special Agreements

No statement contained in this section shall be construed as preventing any special agreement or arrangement between the District and any industrial concern by the District for treatment, subject to payment therefor, by the industrial concern as long as the industrial waste is still in compliance with Federal and State Regulations.

The District may require a User of the sewer services to provide information needed to determine compliance with these Regulations.  All measurements and analyses shall be performed at the User’s expense.  The requirements may include:

1)	Description of wastewaters discharged, together with peak rate and volume over a specified time period.

2) 	Chemical analyses of wastewaters.

3) 	Information on raw materials, processes and products affecting wastewater volume and quality.

4) 	Quantity and disposition of specific liquid, sludge, oil, solvents or other materials important to sewer use control.

5)	A plot plan of sewers of the User’s property showing sewer and pretreatment facility location.

6)	Details of wastewater pretreatment facilities.

7)	Details of systems to prevent and control the losses of materials through spills to the District sewer.

Section 21.  Indemnification

In any and all instances where the District has reasonable cause to believe that any of these Rules and Regulations, or provisions set forth herein or that have otherwise been adopted by the District have been, may be, or are being violated by any User discharging waste into the District wastewater collection and treatment system, in addition or as an the alternative to any enforcement proceedings the District may bring under Article VII, the District may require such Person to give bond or enter into an indemnity agreement in a form acceptable to the District with sufficient surety to protect, indemnify, hold harmless and defend the District from any loss, damage, or expense that may suffer or incur as a result of non-compliance or violation by such user; and, in the event of the failure to do so after ten (10) day notice by registered U. S. Mail that the same will be required.  The use of the District’s wastewater collection and treatment system by such User shall be denied or discontinued.

Article V. – Enforcement and Appeals

Enforcement

A.	Generally.

Any Person who: 1) discharges Sewage, Industrial Wastes or other Wastes into the District Facilities contrary to the provisions of these Rules and Regulations, Federal or State law, regulation, or order or any order or Bylaw of the District, or 2) otherwise violates any provision of these Rules and Regulations, Federal or State law, regulation, or order, or any order or Bylaw of the District, or violates a permit issued hereunder, shall be served by the District with written Notice of Violation, stating the nature of the violation and providing a reasonable time limit for the satisfactory correction thereof.  That Person shall, within the period of time stated in such Notice of Violation, permanently cease or correct all violations.  An explanation of the violation and a plan for satisfactory correction and prevention thereof, to include specific required actions, shall be submitted by that Person to the District during the period provided for correction of the violation.  Submission of this plan in no way relieves the Person of liability for any violations occurring before or after receipt of the Notice of Violation.

If that Person fails to take the required action specified in the Notice of Violation or in the plan submitted by the violator, or the violation continues, the District will take whatever measures necessary to correct or alleviate the violation. Any costs incurred by the District to do so will be at the expense of the Person.

Any Person who shall continue any violation beyond the time limit provided for in the Notice of Violation or refuses to correct or abate the violation shall be fined in the amount not exceeding two thousand five hundred dollars ($1,000) for each violation, with each day of a continuing violation constituting a new violation.  Where multiple sections of these Rules and Regulations are violated each one shall be deemed a separate violation.  The District may institute a civil action to recover fines and costs for violations, and in addition or in the alternative, may seek equitable relief, including but not limited to temporary or permanent injunctive relief.

Any Person violating any of the provisions of these Rules and Regulations, or Federal or State law, regulation, or order, or any order or Bylaw of the District; shall become liable to the District for any expense, loss or damage occasioned the District by reason of such violation, including but not limited to costs and reasonable attorneys’ fees to enforce these Rules and Regulations.  Any such liability shall be in addition to fines and equitable relief sought by the District in connection with a Notice of Violation.

The District may enter into Consent Orders, assurances of voluntary compliance or other similar documents establishing an agreement with any Person responsible for a violation or noncompliance.  Such documents will include specific action to be taken by the Person to correct the violation or noncompliance within a time period specified by the agreement.

Nothing in this section shall limit the authority of the District to take any action, including emergency actions or any other enforcement action, without first issuing a written Notice of Violation.

B.	Permits.

In addition to or in the alternative to the enforcement provisions above, the District may take any or all of the following actions in enforcement of violations of Permits.

C.	Actions and penalties available for Enforcement.

In the conduct of enforcement under Article V, the District may take any or all of the following range of actions and penalties to address a violation of the Permit.

| Action | Application | Description | User Response | Fee |
| --- | --- | --- | --- | --- |
| Letter of Warning | Instance of noncompliance | Letter advising User of instance of non-compliance | Address instance of non-compliance and inform District | None |
| Notice of non-compliance | Significant instances of noncompliance | Notice advising User of instance of non-compliance | Investigation, report, and statement of corrective action | Minimum of $225 per occurrence, to be assessed if User fails to comply with subsequent District Order |
| Notice of Violation | Significant instances of noncompliance or any discharge which threatens SSD and/or general public | Cease and desist order requiring compliance within 90 days | Formal Compliance Plan and Schedule, Interim and Final Compliance Progress Reports | Minimum of $1,000 per occurrence, to be assessed if User fails to comply with subsequent District Order |

D.	Harmful Contributions

The District may suspend the wastewater treatment service and/or a permit when such suspension is necessary, in the opinion of the District, in order to stop an actual or threatened discharge which presents or may present an imminent or substantial endangerment to the health or welfare of the Persons or the environment, causes interference to the District’s Wastewater Collection and Treatment System or causes the District to violate any condition of its MEPDES Permit.

Any Person notified of a suspension of the wastewater treatment service and/or their permit shall immediately stop or eliminate the contribution. In the event of the failure of the Person to comply voluntarily with the suspension order, the District shall take such steps as deemed necessary including immediate severance of the sewage connection, to prevent or minimize damage to the District’s wastewater collection and treatment system or endangerment to any individuals or the environment. The District shall reinstate the permit and/or wastewater treatment service upon statement submitted to the District, by the User within 15 days of the date of the occurrence describing the causes of the harmful contribution and the measures taken to prevent any future occurrence.

E.	Special Charges and Permits

All industrial wastes or industrial wastewaters discharged into the District Facilities under agreement with the District where BOD does not exceed 300 mg/l, where Phosphorus does not exceed 8.97 mg/l, and where the TSS does not exceed 420 mg/l shall be handled by the District for the rates adopted and published annually by the District for normal conveyance and treatment of wastewaters.

All Commercial or Industrial Wastes or Wastewaters discharged into the District Facilities which exceed one or more of the limits above shall pay additional charges at the rate per pound as stated in the District rates published annually.

All Commercial or Industrial Wastes, before being discharged into the District Facilities shall be metered and no waste shall be discharged until so metered. Cost of the meter and all necessary maintenance shall be borne by the User. Appropriate devices or access areas shall be installed and maintained for the purpose of sampling such waste and the District shall have free and continued access over the property of the User for the purpose of sampling the waste.  The District may take such samples at any time or any place as determined by the District.

The District reserves the right to enter into special permits with industrial customers for the collection, treatment and disposal of industrial or sanitary sewage or waste and to enter into permits for the construction and use of special treatment plants under the terms and conditions for the use thereof, and the provisions of this section may be altered, changed, amended or extended under the terms and conditions of such permits.

F.	Termination of Permit

Any User who violates the following conditions of this section, or applicable state and federal regulations, is subject to having its permit terminated in accordance with the procedures set forth in Article V.

1)	Failure of a User to factually report the wastewater constituents and characteristics of its discharge.

2)	Failure of the User to report significant changes in operations, or wastewater constituents and characteristics.

3)	Refusal of reasonable access to the User’s premises for the purpose of inspection or monitoring.

4)	Violation of conditions of the Permit.

G.	Termination of Service

The District may terminate wastewater disposal service and disconnect a pretreatment customer from the system when:

1)	A government agency informs the District that the effluent from the wastewater treatment plant is no longer of quality permitted for discharge to a watercourse, and it is found that the customer is delivering wastewater to the District Facilities that cannot be sufficiently treated or requires treatment that is not provided by the District as normal domestic treatment.

2)	The customer:

a)	Discharges Industrial Waste or Wastewater that is in violation of the Permit with the District;

b) 	Discharges water or wastes having a deleterious effect upon the District’s sewer system;

c) 	Discharges wastewater at an uncontrolled, variable rate in sufficient quantity to cause an imbalance in the wastewater treatment system;

d) 	Repeats a discharge of prohibited wastes to the District’s sewer;

e) 	Fails to pay monthly bills for sewer service when due; or

f)  	Fails to pay all permit charges when due.

The District shall not be held responsible in any way for any damages or inconveniences experienced by the User as a result of termination of service.

H.	Appeals

1)	Appeal to the Superintendent. Any Person aggrieved by any Decision, Permit or Order of the District under these Rules and Regulations, including but not limited to enforcement decisions, may file written objections to the same within thirty (30) calendar days of receipt of the Decision, Permit or Order of the District to the Superintendent of the District. The Superintendent shall reply to the objections within 20 working days of receipt.

2)	Appeal to the Board of Trustees. Decisions by the Superintendent, including any decision on written objections, may be appealed to the Board of Trustees within thirty (30) calendar days of notification of the Decision. The appellant will be given written notice of the time, date and place of the Trustees’ hearing, and may present its appeal at said hearing. A Person aggrieved by the decision of the Board of Trustees may appeal to the Maine Superior Court within thirty (30) calendar days from receipt of the decision of the Trustees.

Article VI – Protection from Damage

No unauthorized Person shall maliciously, willfully or negligently break, damage, destroy, uncover, deface or tamper with any structure, appurtenance or equipment which is a part of the District Facilities.  Any Person violating this provision shall be subject to immediate arrest and punishable by law and or fine by the District.

Article VII.  Powers and Authority of Inspectors.

The Superintendent and other duly authorized employees of the District bearing proper credentials and identification shall be permitted to enter all properties for the purposes of inspection, observation, measurement, sampling and testing in accordance with the provisions of the ordinance.  The Superintendent or his representative shall have no authority to inquire into any processes including metallurgical, chemical, oil, refining, ceramic, paper or other industries beyond that point having a direct bearing on the kind and source of discharge to the sewers or waterways or facilities for wastes treatment.  Any information so obtained and considered as proprietary shall be held so by the District.

While performing the necessary work on private properties referred in this Article, the Superintendent or duly authorized employees of the District shall observe all safety rules applicable to the premises established by the owner or operator of said premises and made available to the District.

The Superintendent and other duly authorized employees of the District bearing proper credentials and identification shall be permitted to enter all private properties through which the District hold a duly negotiated Easement for the purpose of, but not limited to, inspection, observation, measurement, sampling, repair and maintenance of any portion of the District Facilities lying within said Easement.  All entry and subsequent work, if any, on said Easement, shall be done in full accordance with the terms of the duly negotiated easement pertaining to the private property involved.

Article VIII. Severability

If any provision, paragraph, word, section or article of these Rules and Regulations is invalidated by any court of competent jurisdiction, the remaining provision, paragraphs, words, sections, and chapters shall not be affected and shall continue in full force and effect.

Article IX. Validity

All rules and regulations or parts of rules and regulations in conflict herewith are hereby repealed.  The invalidity of any section, clause, sentence or provision of these Rules and Regulations shall not affect the validity of any other part of these Rules and Regulations which can be given effect without such invalid part or parts.

Article X. Effective Date

These Rules and Regulations shall be in full force and effect immediately following its passage, approval, and publication, as provided by law.